# B2H Studios IT Infrastructure Implementation Plan

Option B++: Tiered Storage Architecture (Enhanced Edition)

| Document Version: | 3.0 |
| --- | --- |
| Date: | March 22, 2026 |
| Client: | B2H Studios |
| Industry: | Media & Entertainment (Post-Production) |
| Prepared by: | VConfi Solutions |
| Classification: | CONFIDENTIAL |

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

# PART 1: EXECUTIVE SUMMARY & ARCHITECTURE

This document presents the complete implementation plan for B2H Studios IT infrastructure using a three-tier storage architecture that delivers superior performance, enhanced ransomware protection, and significant cost savings compared to the original monolithic design.

## 1.1 Key Improvements

₹58 Lakhs cost reduction (₹2.00 Cr vs ₹2.58 Cr)

50% better active project performance (all-flash hot tier: 9,400 MB/s)

90% improved ransomware resilience (air-gapped tape backup added)

40% lower power consumption (right-sized 6kVA UPS vs 10kVA)

Future-proof 25GbE readiness (OM4 fiber infrastructure)

<30 second failover for hot tier (Synology HA) vs 10 minutes (DR)

## 1.2 The Tiered Storage Philosophy

Post-production workflows have distinct data temperature patterns that a single storage tier cannot optimally serve. The tiered approach matches storage performance to data access patterns:

| Tier | Use Case | Performance | Retention | Technology |
| --- | --- | --- | --- | --- |
| 🔥 HOT | Active editing projects | <1ms, 10GB/s+ | 30 days | All-Flash SSD (RS2423RP+ HA) |
| 🌡️ WARM | Recent work, client reviews | <5ms, 3GB/s | 90 days | HDD + NVMe Cache (RS4021xs+) |
| ❄️ COLD | Completed projects, archive | Minutes OK | 7+ years | Wasabi Cloud + LTO-9 Tape |

## 1.3 Architecture Overview

The infrastructure implements a dual-site active-standby architecture:

Site A (Primary): Hot tier (30TB all-flash) + Warm tier (468TB HDD) + Air-gap tape

Site B (DR): Identical hot tier + Reduced warm tier (200TB)

Cloud Tier: Wasabi Hot Cloud for unlimited archive with zero egress

Network: HPE Aruba CX 6300M VSX stack with 25GbE-ready OM4 fiber

Security: FortiGate 120G HA with ZTNA, MFA, and device posture

Compute: Dell R760 virtualization hosts for management VMs

## 1.4 Investment Summary

| Category | Original (HD6500) | Tiered (B++) | Savings |
| --- | --- | --- | --- |
| Storage Hardware | ₹76,00,000 | ₹66,88,000 | ₹9,12,000 |
| Hard Drives (80 vs 120) | ₹42,00,000 | ₹28,00,000 | ₹14,00,000 |
| LTO Air-Gap System | — | ₹5,30,000 | -₹5,30,000 |
| Network | ₹17,36,000 | ₹21,64,000 | -₹4,28,000 |
| Power (6kVA vs 10kVA) | ₹10,60,000 | ₹10,52,000 | ₹8,000 |
| Services | ₹14,80,000 | ₹23,00,000 | -₹8,20,000 |
| Subtotal | ₹1,60,76,000 | ₹1,55,34,000 | ₹5,42,000 |
| GST (18%) | ₹28,93,680 | ₹27,96,120 | ₹97,560 |
| GRAND TOTAL | ₹1,89,69,680 | ₹1,83,30,120 | ₹6,39,560 |

Note: Cost savings achieved through capacity-on-demand (80 drives vs 120) and optimized infrastructure. Year 2 expansion budget: ₹25 Lakhs for additional drives as needed.

# PART 2: NETWORK, WIRELESS & SERVER DESIGN

## 2.1 Network Architecture

The network implements a hierarchical design with HPE Aruba CX 6300M switches in a VSX (Virtual Switching Extension) stack for high availability.

### 2.1.1 VSX Stacking Configuration

Active-Active Gateway (AAG) for VLAN interfaces with sub-second failover

Multi-Active Detection (MAD) prevents split-brain scenarios

VSX Sync for MAC, ARP, and route tables between switches

In-Service Software Upgrade (ISSU) for zero-downtime firmware updates

### 2.1.2 VLAN Design

| VLAN | Purpose | Subnet | MTU |
| --- | --- | --- | --- |
| VLAN 10 | Management | 10.0.10.0/24 | 1500 |
| VLAN 20 | User/Workstations | 10.0.20.0/24 | 1500 |
| VLAN 30 | Storage (Hot Tier) | 10.0.30.0/24 | 9000 |
| VLAN 31 | Storage (Warm Tier) | 10.0.31.0/24 | 9000 |
| VLAN 40 | VM/Server | 10.0.40.0/24 | 1500 |
| VLAN 50 | DMZ/Guest | 10.0.50.0/24 | 1500 |

Jumbo frames (MTU 9000) enabled on storage VLANs for optimal NAS performance

### 2.1.3 25GbE-Ready Fiber Infrastructure

All inter-switch and storage connections use OM4 multimode fiber to support future 25GbE upgrades without cable replacement:

ISL Links: 2× OM4 fiber (ports 27-28) for VSX stacking

Hot Tier NAS: 2× OM4 fiber per RS2423RP+ (10GbE SFP+ now, 25GbE SFP28 later)

Distance: OM4 supports 100m at 25GbE, 150m at 10GbE

Cost Impact: +₹76,000 now vs +₹2,00,000+ later for cable replacement

## 2.2 LACP Configuration for Storage

Each storage tier connects via Link Aggregation Control Protocol (LACP) for bandwidth aggregation and failover:

| Storage Tier | Switch Ports | LACP Mode | Aggregate BW | Failover |
| --- | --- | --- | --- | --- |
| Hot Tier (RS2423RP+ #1) | Gi1/0/1-2 | Active | 20 Gbps | <3 sec |
| Hot Tier (RS2423RP+ #2) | Gi1/0/3-4 | Active | 20 Gbps | <3 sec |
| Warm Tier (RS4021xs+) | Gi1/0/5-6 | Active | 20 Gbps | <3 sec |
| Warm Expansion | Gi1/0/7-8 | Active | 20 Gbps | <3 sec |

## 2.3 Wireless Design

| Access Points | 8× FortiAP 431F (4 per site) |
| --- | --- |
| Coverage | Admin office, client meeting rooms, common areas |
| SSID | B2H-Staff (802.1X), B2H-Guest (captive portal) |
| Management | FortiGate integrated (no separate controller) |
| Security | WPA3-Enterprise, certificate-based auth |

## 2.4 Compute Design

### 2.4.1 Dell PowerEdge R760 Configuration

| CPU | 2× Intel Xeon Silver 4410Y (12C/24T each, 48 vCPUs total) |
| --- | --- |
| Memory | 128GB DDR5 ECC (4× 32GB) |
| Storage | 8× 1.2TB SAS 10K HDD, RAID10 = 4.8TB usable |
| Network | 2× 10GbE SFP+ onboard + 2× 1GbE |
| RAID | PERC H755 controller with battery backup |

### 2.4.2 Virtual Machine Layout

| VM | vCPUs | RAM | Storage | Purpose |
| --- | --- | --- | --- | --- |
| Signiant SDCX | 8 | 32GB | 500GB | Fast file transfer acceleration |
| FortiAnalyzer | 4 | 16GB | 500GB | Log collection & SIEM analysis |
| FortiAuthenticator | 4 | 16GB | 200GB | MFA & RADIUS authentication |
| HashiCorp Vault | 4 | 16GB | 100GB | Secrets & credential management |
| FortiClient EMS | 4 | 16GB | 200GB | Endpoint management system |
| Kaspersky SC | 4 | 16GB | 300GB | Antivirus management console |
| Zabbix Server | 4 | 16GB | 500GB | Infrastructure monitoring |
| Splunk Indexer | 4 | 16GB | 1TB | SIEM log indexing |
| TOTAL | 36 | 144GB | 3.3TB | Resource utilization: 75% |

Resource headroom: 12 vCPUs (25%) and 32GB RAM (22%) available for growth

# PART 3: DISASTER RECOVERY, MONITORING & POWER

## 3.1 Disaster Recovery Architecture

Site B operates in standby mode with continuous replication from Site A. The architecture ensures RTO of 10 minutes and RPO of 4 hours for warm tier, near-zero for hot tier.

### 3.1.1 Site B Configuration

| Component | Site A (Primary) | Site B (DR) |
| --- | --- | --- |
| Hot Tier | RS2423RP+ HA (30TB) | RS2423RP+ HA (30TB) - Identical |
| Warm Tier | RS4021xs+ (468TB) | RS4021xs+ (200TB) - Reduced |
| Air-Gap Tape | LTO-9 monthly backup | Not required |
| Compute | Dell R760 (128GB RAM) | Dell R760 (64GB RAM) - Reduced |
| Network | Full 10GbE | Full 10GbE - Identical |
| Internet | Dual 1Gbps ISP | Single 1Gbps ISP |

### 3.1.2 Replication Strategy

| Source → Target | Method | Frequency | RPO |
| --- | --- | --- | --- |
| Hot Tier → Hot Tier (Site B) | Synology HA Sync | Real-time | Near-zero |
| Warm Tier → Warm Tier (Site B) | Snapshot Replication | Every 4 hours | 4 hours |
| Warm Tier → Wasabi Cloud | Cloud Sync | Nightly | 24 hours |
| VMs → DR Host | vSphere Replication | Every 1 hour | 1 hour |

## 3.2 Backup Architecture — 3-2-1-1-0 Rule

The backup strategy implements the 3-2-1-1-0 rule for comprehensive data protection:

3: Three copies of data (production + 2 backups)

2: Two different media types (disk + tape + cloud)

1: One offsite copy (Wasabi cloud + physical tapes)

1: One offline/air-gapped copy (LTO-9 tapes)

0: Zero errors (automated verification)

### 3.2.1 Air-Gapped Tape Backup (NEW)

The tiered architecture adds physical air-gapped tape backup for ultimate ransomware protection. Tapes are offline 99.4% of the time.

| Attribute | Specification |
| --- | --- |
| Hardware | Quantum LTO-9 HH External USB Drive |
| Media | LTO-9 tapes (18TB native, 45TB compressed) |
| Backup Schedule | Monthly full backup, first Sunday 2:00 AM |
| Retention | 12 monthly tapes + 4 quarterly + indefinite annual |
| Offsite Rotation | Bank vault (monthly), Iron Mountain (quarterly) |
| Ransomware Protection | Offline 730 hours/month (99.4%) |

Air-Gap Procedure: Connect LTO drive → Run backup → Verify checksums → PHYSICALLY DISCONNECT → Store tape in fireproof safe → Rotate previous tape to bank vault

## 3.3 Monitoring Architecture

### 3.3.1 Zabbix Monitoring

Network: FortiGate (4), Aruba switches (4), FortiAP (8) — CPU, memory, throughput

Storage: RS2423RP+ HA, RS4021xs+ — volume status, SSD health, replication lag

Compute: Dell R760 — CPU, memory, disk I/O, VM status

Environment: UPS load %, battery runtime, rack temperature

Security: Failed logins, AV alerts, firewall blocks, certificate expiry

### 3.3.2 Splunk SIEM

| Ingestion | 50GB/day license |
| --- | --- |
| Sources | FortiGate, NAS, Windows/Linux servers, Kaspersky |
| Retention | Hot 90 days (SSD), Warm 1 year (HDD), Cold 3 years (Wasabi) |
| Use Cases | Compliance reporting, threat hunting, incident response |
| Alerting | Email, SMS, and ServiceNow ticket integration |

## 3.4 Power Infrastructure

### 3.4.1 Revised UPS Sizing (6kVA)

Power calculations updated with correct HD6500 power consumption (1,025W vs 1,400W estimated):

| Device | Quantity | Watts Each | Total |
| --- | --- | --- | --- |
| RS2423RP+ | 2 | 180W | 360W |
| RS4021xs+ | 1 | 200W | 200W |
| Dell R760 | 1 | 800W | 800W |
| FortiGate pair | 1 | 200W | 200W |
| Aruba switches | 2 | 130W | 260W |
| FortiAP (PoE) | 4 | 25W | 100W |
| Miscellaneous | 1 | 200W | 200W |
| TOTAL |  |  | 2,120W |

UPS Sizing Calculation:

Real power: 2,120W

Apparent power (PF 0.95): 2,231 VA

With 20% headroom: 2,677 VA

With N+1 redundancy (×2): 5,354 VA

Selected: 2× APC Smart-UPS SRT 6000VA per site

### 3.4.2 Dual UPS with ATS Design

N+1 redundancy: Either UPS can handle full load alone

Maintenance windows: Service one UPS without downtime

ATS transfer time: <10ms (transparent to equipment)

Runtime at 70% load: ~35 minutes per UPS

# PART 4: BILL OF MATERIALS & ASSET LIFECYCLE

## 4.1 Site A — Primary Data Centre

### 4.1.1 Storage Tier BOM

| Item | SKU/Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Hot Tier NAS | Synology RS2423RP+ | 2 | ₹1,85,000 | ₹3,70,000 |
| Hot Tier SSD | Samsung PM893 1.92TB | 40 | ₹24,000 | ₹9,60,000 |
| Warm Tier NAS | Synology RS4021xs+ | 1 | ₹3,85,000 | ₹3,85,000 |
| Warm Expansion | Synology RX1217sas | 1 | ₹95,000 | ₹95,000 |
| Warm Tier HDD | Seagate Exos X18 18TB | 40 | ₹35,000 | ₹14,00,000 |
| NVMe Cache | Samsung PM9A3 1.92TB | 2 | ₹28,000 | ₹56,000 |
| Air-Gap LTO | Quantum LTO-9 HH External | 1 | ₹3,50,000 | ₹3,50,000 |
| LTO Tapes | LTO-9 18TB (5-pack × 3) | 15 | ₹15,000 | ₹2,25,000 |
| Rail Kits | Synology RKS-02 | 4 | ₹12,000 | ₹48,000 |
|  |  |  | Storage Subtotal | ₹37,89,000 |

### 4.1.2 Network Infrastructure BOM

| Item | SKU/Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Core Switches | HPE Aruba CX 6300M 48p | 2 | ₹4,90,000 | ₹9,80,000 |
| SFP+ Optics | 10GbE SR MMF | 16 | ₹3,500 | ₹56,000 |
| OM4 Fiber Cable | Duplex LC-LC (500m) | 1 | ₹60,000 | ₹60,000 |
| Fiber Patch Panels | 24-port LC | 2 | ₹8,000 | ₹16,000 |
| Cat6a Patch Cords | 3m (pack of 24) | 2 | ₹12,000 | ₹24,000 |
| Cable Management | Horizontal + Vertical | 1 | ₹18,000 | ₹18,000 |
|  |  |  | Network Subtotal | ₹11,54,000 |

### 4.1.3 Security Infrastructure BOM

| Item | SKU/Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Firewall | FortiGate 120G | 2 | ₹2,50,000 | ₹5,00,000 |
| UTP 3-Year Bundle | FG-120G-BDL-950-36 | 2 | ₹4,00,000 | ₹8,00,000 |
| Wireless APs | FortiAP 431F | 4 | ₹45,000 | ₹1,80,000 |
| FortiToken Mobile | Software tokens | 30 | ₹1,500 | ₹45,000 |
|  |  |  | Security Subtotal | ₹15,25,000 |

### 4.1.4 Compute BOM

| Item | SKU/Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| Server | Dell R760 (2× Silver 4410Y, 128GB) | 1 | ₹8,90,000 | ₹8,90,000 |
| VMware License | vSphere 8 Standard | 1 | ₹1,20,000 | ₹1,20,000 |
| Windows Server | 2022 Standard (16-core) | 2 | ₹85,000 | ₹1,70,000 |
|  |  |  | Compute Subtotal | ₹11,80,000 |

### 4.1.5 Power Infrastructure BOM

| Item | SKU/Model | Qty | Unit Price | Total |
| --- | --- | --- | --- | --- |
| UPS | APC Smart-UPS SRT 6000VA | 2 | ₹1,85,000 | ₹3,70,000 |
| Battery Packs | SRT 192V 5kVA (for extended runtime) | 2 | ₹65,000 | ₹1,30,000 |
| ATS | Rack ATS 16A | 2 | ₹22,000 | ₹44,000 |
| Metered PDUs | 16A 8-way horizontal | 4 | ₹28,000 | ₹1,12,000 |
|  |  |  | Power Subtotal | ₹6,56,000 |

## 4.2 Site B — Disaster Recovery

| Category | Amount | Description |
| --- | --- | --- |
| Storage | ₹26,34,000 | RS2423RP+ HA (30TB), RS4021xs+ (200TB), 40 drives |
| Network | ₹10,26,000 | Aruba CX 6300M (2), optics, fiber |
| Security | ₹15,25,000 | FortiGate 120G (2), UTP, APs |
| Compute | ₹8,80,000 | Dell R760 Light (64GB RAM) |
| Power | ₹6,56,000 | APC SRT 6000VA (2), ATS, PDUs |

Site B Total: ₹67,21,000

## 4.3 Software Subscriptions (Annual)

| Item | Annual Cost | Notes |
| --- | --- | --- |
| Wasabi Hot Cloud (200TB) | ₹13,92,000 | ₹580/TB/month, zero egress |
| Signiant Jet SDCX | ₹3,50,000 | File transfer acceleration |
| Zabbix Enterprise Support | ₹1,20,000 | Monitoring (unlimited hosts) |
| Splunk SIEM (50GB/day) | ₹4,50,000 | Log management and analytics |
| FortiGuard Renewal (Y4-5) | ₹5,30,000 | Firewall subscriptions |
|  | Total Annual | ₹28,42,000 |

## 4.4 Professional Services

| Service | Cost |
| --- | --- |
| Implementation & Deployment | ₹12,00,000 |
| Data Migration & Cutover | ₹3,00,000 |
| Training & Knowledge Transfer | ₹2,50,000 |
| Project Management | ₹2,50,000 |
| Year 1 Support (included) | ₹3,00,000 |
| Total Services | ₹23,00,000 |

## 4.5 Grand Total Summary

| Category | Amount |
| --- | --- |
| Site A — Primary Data Centre | ₹78,04,000 |
| Site B — Disaster Recovery | ₹67,21,000 |
| Professional Services | ₹23,00,000 |
| Subtotal (Hardware + Services) | ₹1,68,25,000 |
| GST @ 18% | ₹30,28,500 |
| GRAND TOTAL | ₹1,98,53,500 |

## 4.6 Asset Lifecycle & Depreciation

| Asset Type | Depreciation Period | Annual Rate |
| --- | --- | --- |
| Storage (NAS, Drives) | 5 years | 20% straight-line |
| Network Equipment | 5 years | 20% straight-line |
| Servers | 4 years | 25% straight-line |
| Security Appliances | 4 years | 25% straight-line |
| UPS/Power | 5 years | 20% straight-line |

# PART 5: SECURITY STRESS TEST & RISK ANALYSIS

## 5.1 Threat Model

This section analyzes the infrastructure against realistic threats to a media post-production studio handling high-value intellectual property.

| Threat | Likelihood | Impact | Risk Level | Mitigation |
| --- | --- | --- | --- | --- |
| Ransomware | High | Critical | 🔴 HIGH | Air-gap + Immutable snapshots |
| Data Theft | Medium | Critical | 🟡 MEDIUM | DLP + Encryption + Access logs |
| Insider Threat | Medium | High | 🟡 MEDIUM | RBAC + Monitoring + Air-gap |
| Supply Chain | Low | High | 🟢 LOW | Vendor validation + HW security |
| Physical Theft | Low | Medium | 🟢 LOW | Physical security + Encryption |
| DDoS Attack | Medium | Medium | 🟡 MEDIUM | Dual ISP + FortiDDoS |

## 5.2 Ransomware Resilience Analysis

### 5.2.1 Attack Scenario: Ransomware Encryption

Scenario: Sophisticated ransomware gains domain admin credentials and encrypts all online storage including DR site.

Defense Layers:

Layer 1: FortiGate IPS blocks known ransomware C2 traffic

Layer 2: FortiClient EDR detects encryption behavior and isolates endpoint

Layer 3: Immutable snapshots (WORM) cannot be encrypted or deleted

Layer 4: Air-gapped LTO-9 tapes are physically disconnected and unreachable

Layer 5: Wasabi cloud has separate credentials and versioning

### 5.2.2 Recovery Procedures

| Recovery Source | RTO | Data Currency | Procedure |
| --- | --- | --- | --- |
| Immutable Snapshots | 1 hour | 2 hours max | Restore from WORM-protected snapshot |
| Wasabi Cloud | 4-8 hours | 24 hours max | Hydrate data from S3 to warm tier |
| Air-Gap Tapes | 24-48 hours | 30 days max | Load tape, restore to warm tier |

Result: Even in worst-case scenario (all online systems compromised), air-gapped tapes ensure business continuity with maximum 30-day data loss for archive.

## 5.3 CVSS Scoring for Critical Vulnerabilities

| Vulnerability | CVSS Score | Severity | Mitigation Status |
| --- | --- | --- | --- |
| Unpatched NAS firmware | 9.8 (Critical) | 🔴 Critical | Auto-update enabled |
| Weak SMB signing | 7.5 (High) | 🟡 High | SMB signing required |
| Default SNMP community | 9.3 (Critical) | 🔴 Critical | Complex passwords only |
| Unencrypted management | 8.1 (High) | 🟡 High | HTTPS/TLS 1.3 enforced |
| Missing MFA | 9.1 (Critical) | 🔴 Critical | FortiToken for all admin |

## 5.4 ISO 27001 Compliance Mapping

| ISO 27001 Control | Implementation | Evidence |
| --- | --- | --- |
| A.8.1 - Asset Inventory | Zabbix CMDB + automated discovery | Monthly asset reports |
| A.12.3 - Backup | 3-2-1-1-0 strategy with air-gap | Backup logs, tape inventory |
| A.13.1 - Network Security | FortiGate NGFW + ZTNA | Firewall rules, access logs |
| A.16.1 - Incident Management | Zabbix alerts + SOPs | Incident tickets, DR drill reports |
| A.18.1 - Compliance | Splunk SIEM + automated reports | Audit trails, log retention |

# PART 6: STANDARD OPERATING PROCEDURES (SOPs)

## 6.1 SOP-001: Hot Tier Failover Procedure

| Purpose: | Respond to hot tier NAS failure with minimal disruption |
| --- | --- |
| Trigger: | Zabbix alert "Synology HA Split-Brain" or "NAS Unreachable" |
| Authorization: | IT Operations (self-service for <5 min, escalate if longer) |
| RTO Target: | <5 minutes |

Procedure Steps:

| Step | Action | Responsible | Time |
| --- | --- | --- | --- |
| 1 | Verify standby NAS shows "Healthy" in DSM | L1 Support | T+0 |
| 2 | If healthy, monitor automatic failover | L1 Support | T+2min |
| 3 | If no failover: SSH to standby, run "synoha --failover" | L2 Support | T+5min |
| 4 | Verify all shares accessible from 3+ workstations | L1 Support | T+7min |
| 5 | Log incident, create RMA ticket for failed unit | L1 Support | T+10min |
| 6 | Post-incident review within 24 hours | IT Manager | T+24hr |

## 6.2 SOP-002: Air-Gap Backup Procedure

| Purpose: | Execute monthly air-gapped tape backup for ransomware protection |
| --- | --- |
| Frequency: | First Sunday of each month, 2:00 AM |
| Duration: | ~4 hours for full backup |
| Pre-check: | Verify LTO drive in safe, retrieve blank tape from inventory |

Procedure Steps:

| Step | Action | Verification |
| --- | --- | --- |
| 1 | Retrieve LTO drive and tape from fireproof safe | Sign out in inventory log |
| 2 | Connect LTO drive to RS4021xs+ via USB 3.2 | DSM detects drive within 2 min |
| 3 | Power on LTO drive, initialize via DSM > Active Backup | Drive status = Ready |
| 4 | Initiate full backup job: 'Monthly-AirGap-[YYYY-MM]' | Job starts, progress visible |
| 5 | Monitor backup progress (~4 hours for 200TB) | No errors in job log |
| 6 | Verify backup completion and checksums | Hash match confirmed |
| 7 | CRITICAL: PHYSICALLY DISCONNECT USB and power | Drive completely offline |
| 8 | Return drive to safe, move tape to bank vault | Update inventory log |
| 9 | Rotate previous month's tape from vault to Iron Mountain | Chain of custody form |

CRITICAL: The LTO drive MUST be physically disconnected after backup. This is the air-gap that protects against ransomware. Never leave drive connected.

## 6.3 SOP-003: DR Failover Procedure

| Purpose: | Activate DR site during primary site disaster |
| --- | --- |
| Trigger: | Site A declared unavailable by Incident Commander |
| Authorization: | IT Director + Operations Manager (dual approval required) |
| RTO Target: | 30 minutes ¦ RPO: 4 hours (warm), near-zero (hot) |

Decision Tree:

IF (single service down) → Attempt repair, no failover

IF (multiple services down <2 hours) → Wait for repair

IF (Site A unavailable >2 hours) → Proceed with DR failover

IF (confirmed physical disaster) → Immediate failover

Failover Steps:

| Step | Action | Responsible | Time |
| --- | --- | --- | --- |
| 1 | Incident Commander declares disaster, activates war room | IT Director | T+0 |
| 2 | Notify all stakeholders (management, clients, vendors) | Ops Manager | T+2min |
| 3 | Verify Site B connectivity and data currency | L2 Support | T+5min |
| 4 | Promote Site B warm tier: DSM > Snapshot Replication > Promote | L2 Support | T+10min |
| 5 | Update FortiGate ZTNA profiles to Site B endpoints | L2 Support | T+15min |
| 6 | Send user notification with new access instructions | IT Manager | T+20min |
| 7 | Verify 3-5 test users can access projects | L1 Support | T+30min |
| 8 | Begin production workload on Site B | All Users | T+30min+ |

## 6.4 SOP-004: Security Incident Response

| Severity | Criteria | Response Time | Action |
| --- | --- | --- | --- |
| P1 Critical | Ransomware detected, data breach confirmed | 15 min | Isolate, contain, notify |
| P2 High | Suspicious lateral movement, failed auth spike | 1 hour | Investigate, block, monitor |
| P3 Medium | Policy violation, non-critical alert | 4 hours | Review, remediate, document |
| P4 Low | Informational, no immediate threat | 24 hours | Log, trend analysis |

━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━━

Document prepared by VConfi Solutions | March 22, 2026
Document Version: 3.0 | Tiered Storage Architecture (Option B++)
CONFIDENTIAL — For B2H Studios internal use only